Readme for 82578 GbE NVM

Care should be taken to use the correct GbE NVM (Non-Volatile Memory) firmware revision for the stepping combination of the Ibex Peak and the 82578. The table below lists the NVM revision that is optimized for use with the silicon stepping combination.

NOTE: Using a newer revision NVM with an older silicon stepping or older revision NVM with a newer silicon stepping could cause system instability and unpredictable behavior.

| IBX Stepping | 82578 Stepping/PHY-Ver1 | Corporate (82578DM) or Consumer (82578DC) Sku | NVM Version | Comments |
| --- | --- | --- | --- | --- |
| B2/B3 | C0 = 0x42 | DM | 0.C2 | Please do not forget to upgrade to Release 14.6 Driver version 11.2.19.0 |
|  |  | DC | 0.C5 |  |

1. See below for detail on PHY versions.

How to identify the PHY stepping in the system?

Use the latest version of LANCONF (Intel LAN tool found in the SVK (Sample Validation Kit)).  Navigate to Registers MAC and PHY Registers -> PHY Registers Menu -> Raw PHY Registers.  On the Raw PHY Registers screen, reference the value of the lower byte of register 0x0003. A value of 0x41 indicates B-0 stepping and 0x42 indicates C-0 stepping.

How to identify the Ibex Peak stepping?

Use a PCI tool to read the LPC Device ID D31:f0 offset 08h. If the byte value in the field reads:

02h: IBX A1 stepping

03h: IBX B0 stepping

04h: IBX B1 stepping

05h: IBX B2 stepping